How to connect to your mqtt server

Introduction

The firmware installed on the device defines a fixed format for themes and subscription message formats. Default connection to the tms and emxq mqtt servers of dspread. You can quickly connect your device to your MQTT server by updating parameters through the TMS system.

TMS login

Please contact dspread to obtain tms login permission

Create Cert and Param OTA package

- About tms/CertParam/ ResourceFile/posparam.ini

posparam.ini is JSON fomart.

The following is a list of parameters that support remote updates

| Param Name | Type |  |
| --- | --- | --- |
| MqttUrl | String | MQTT server address |
| MqttPort | String | MQTT server port |
| username | String | Username for connecting to MQTT server |
| password | String | Password for connecting to MQTT server |
| clietId | String | ClientId for connecting to MQTT server In general, no setting is required, SN is used as the client ID by default in the firmware |
| Topic | String | It is the base topic, and the device will automatically append its own SN to form the final subscription topic Example;user/message/sn |
| LogStatus | Int | 0: Release 2:Debug |
| CertState | Int | 0: No certificate needs to be set up 1: A certificate needs to be set up |

- About original Cert and Param files

If set CertState with 0,

posparam.in file is needed to create CertParam_V1.0.6.zip

Please note that the zip file only contains the posparam.ini file Cannot contain other folders

If set CertSate with 1 posparam.iniand cert files are needed to create CertParam_V1.0.6.zip

- Open powershell and run ./CertParam/package_resource.ps1

- Create Cert and Param OTA package.

Update parameters

Please check “how to use tms.docx”

Topic and Message Format

The default firmware subscription topic and message formats are as follows.

Topic: user/message/TDS10SEU000030

Please note that TDS10SEU000030 is the device serial number, you need to modify it to your device serial number

Message Format:

{ "status": "success","orderId":"12345677","amount":1234.56 }

Please note that orderId cannot be duplicated