DS10 API Description

V 1.0.0

- Writing purpose

This document is used to describe the API usage of Dinghe Cloud Speaker SDK, enabling programmers to quickly carry out secondary development.。

- Platform

The API described in this document is applicable to the Dinghe Cloud Speaker Mobile EC600X platform.

- API

  - Streaming API

ql_set_volume

|  | void ql_set_volume(AUDIOHAL_SPK_LEVEL_T vol); |
| --- | --- |
| Func | Set system volume |
| Param | vol: The volume size to be set, refer to the enumeration AUDIOHAL_SPK_LEVEL_T |
| Return | VOID |

ql_get_volume

|  | int ql_get_volume(void); |
| --- | --- |
| Func | Get the current system volume |
| Param | VOID |
| Return | Current volume level |

tts_play_immediately

|  | int tts_play_immediately(const char* buf,int len, tts_text_type type); |
| --- | --- |
| Func | Audio broadcast message release function, searches for files through strings and plays them (high priority, plays immediately, queue number is one, only supports storing one broadcast message) |
| Param | buf Index of the broadcasted string len string length type Broadcast type (using FILE_TEXT, supports single file) |
| Return | 0 succeed 1 error |

tts_play_set

|  | int tts_play_set(const char* buf,int len, tts_text_type tts_type); |
| --- | --- |
| Func | Audio broadcast message release function, searches for files through strings and plays them (low priority, queue size of 50, only supports storing 50 broadcast messages) PS: Broadcast multiple file combinations, such as payment message broadcast, type AMTtEXT. The function called is mqtt_play_analy, as shown in the table below |
| Param | buf Index of the broadcasted string len buf length type Broadcast type (using FILE_TEXT, supports single file) |
| Return | 0 succeed 1 error |

mqtt_play_analy

|  | int mqtt_play_analy(struct tts_buff_msg * reviceMsg); |
| --- | --- |
| Func | The fixed format for parsing the amount message broadcast is (payment institution+amount+unit) |
| Param | reviceMsg The structure of the incoming voice message, the specific content depends on the definition of the structure tts_buff_msg |
| Return | 0 succeed 1 error |

q tts_play_local_file

|  | tts_play_local_file(file_root_path); |
| --- | --- |
| Func | Audio file playback, playing audio files from the file system in WAV, MP3 formats |
| Param | file_root_path file name |
| Return | 0 succeed 1 error |

ql_mp3_file_play

|  | int ql_mp3_file_play(char *file_name); |
| --- | --- |
| Func | MP3 file playback |
| Param | file_name file path |
| Return | 0 succeed other error |

ql_mp3_file_play2

|  | int ql_mp3_file_play2(char *data, unsigned int size); |
| --- | --- |
| Func | MP3 streaming playback |
| Param | data Stream data pointer size length |
| Return | 0 succeed other error |

ql_mp3_file_stop

|  | void ql_mp3_file_stop(void); |
| --- | --- |
| Func | MP3 stream playback stopped |
| Param | VOID |
| Return | VOID |

ql_wav_file_play

|  | int ql_wav_file_play (char *file_name); |
| --- | --- |
| Func | MAV file playback |
| Param | file_name file path |
| Return | 0 succeed other error |

ql_wav_file_play2

|  | int ql_wav_file_play2(char *data, unsigned int size); |
| --- | --- |
| Func | WAV streaming playback |
| Param | data Stream data pointer size length |
| Return | 0 succeed other error |

ql_wav_file_stop

|  | void ql_wav_file_stop(void); |
| --- | --- |
| Func | WAV stream playback has stopped |
| Param | VOID |
| Return | VOID |

ql_amr_file_play

|  | int ql_amr_file_play(char *file_name); |
| --- | --- |
| Func | AMR file playback |
| Param | file_name file path |
| Return | 0 succeed other error |

ql_amr_file_play2

|  | int ql_amr_file_play2(char *data, unsigned int size); |
| --- | --- |
| Func | AMR streaming playback |
| Param | data data pointer size data length |
| Return | 0 succeed other error |

ql_amr_file_stop

|  | void ql_amr_file_stop(void); |
| --- | --- |
| Func | AMR stream playback stopped |
| Param | VOID |
| Return | VOID |

  - 设备信息获取

ql_dev_get_cpu_uid

|  | void ql_dev_get_cpu_uid(unsigned long long * uid_ret); |
| --- | --- |
| Func | Get cpu_uid |
| Param | uid_ret: Used for saving device's cpu_uid |
| Return | VOID |

sysparam_get

|  | struct sysparam *sysparam_get(void); |
| --- | --- |
| Func | Retrieve the current system parameters, which can be defined in systemparam. h. The storage nature is to shut down without loss |
| Param | VOID |
| Return | sysparam Pointer to device information NULL error |

ql_dev_get_imei

|  | QL_DEV_ERROR_CODE ql_dev_get_imei(char *imei,unsigned int imei_len); |
| --- | --- |
| Func | Obtain the IMEI of the device. |
| Param | imei：IMSI of the device, string imei_len：imei length |
| Return | QL_DEV_SUCCESS succeed QL_DEV_GENERIC_FAILURE error |

ql_dev_get_mac

|  | int ql_dev_get_mac(unsigned char *mac_val,unsigned int len); |
| --- | --- |
| Func | Obtain the MAC number of the customer's device (17 bytes) |
| Param | V mac_val：Store MAC buffer, with at least 17 bytes of incoming buffer. version_len：MAC number length; For 17 bytes |
| Return | 0 succeed other error |

Module_Net_State

|  | int Module_Net_State(void); |
| --- | --- |
| Func | Get 4G network status |
| Param | VOID |
| Return | 0: NET DISCON 1:NET CON |

ql_sim_get_card_statu

|  | QL_SIM_ERROR_CODE ql_sim_get_card_status(int *card_status); |
| --- | --- |
| Func | Get SIM card status information immediately |
| Param | card_status：Return SIM card LEI type information, see QL_SIM-STRUS |
| Return | QL_SIM_SUCCESS succeed QL_SIM_GENERIC_FAILURE error |

ql_nw_get_signal_strength

|  | QL_NW_ERROR_CODE ql_nw_get_signal_strength(QL_NW_SIGNAL_STRENGTH_INFO_T *pt_info); |
| --- | --- |
| Func | Obtain detailed signal strength |
| Param | pt_info Return detailed signal strength |
| Return | 0 succeed other error |

battery_read_percent

|  | uint8_t battery_read_percent(void); |
| --- | --- |
| Func | Obtain battery level |
| Param | VOID |
| Return | 0 to 100 Corresponding percentage 0： 0% 100：100% |

ql_rtos_get_free_heap_size

|  | int ql_rtos_get_free_heap_size(); |
| --- | --- |
| Func | Get the user's available memory size |
| Param | VOID |
| Return | memory size |

  - Device Reboot

ql_power_reset

|  | void ql_power_reset(); |
| --- | --- |
| Func | Restart module (will wait for RF to be turned off before restarting) |
| Param | VOID |
| Return | VOID |

  - HTTP

Ext_Wifi_AppOta

|  | int Ext_Wifi_AppOta(char * url); |
| --- | --- |
| Func | OTA package download and verification |
| Param | VOID |
| Return | 0：succeed other：error |

Gprs_HTTPC_PostB

|  | int Gprs_HTTPC_PostB(HTTPParam *httpinfo, const char * const * HeadAdd,CHAR *Buffer,INT32 bufSize, unsigned int *recvSize); |
| --- | --- |
| Func | HTTP post request sending and server response receiving |
| Param | httpinfo The information sent includes the URL and the content being sent headAdd Head information Buffer Pointer for receiving data bufSize Maximum length received recvSize The actual length returned |
| Return | 0 succeed other error |

  - LED

LedModeSet

|  | static void LedModeSet(int LedSel,LedStatSet Mode); |
| --- | --- |
| Func | LED light mode setting |
| Param | LedSel color set BLUE_LED_SET GREEN_LED_SET RED_LED_SET Mode mode set Blink Alternating flashes FlashFast flash mob FlashSlow slow flash AlwaysOn always on AlwaysOff always off |
| Return | VOID |

setRedLedOnOff

|  | void setRedLedOnOff(uint8_t on); |
| --- | --- |
| Func | Red light display control |
| Param | 1 on 0 off |
| Return | VOID |

setBlueLedOnOff

|  | void setBlueLedOnOff(uint8_t on); |
| --- | --- |
| Func | Blue light display control |
| Param | 1 on 0 off |
| Return | VOID |

setGreenLedOnOff

|  | void setGreenLedOnOff(uint8_t on); |
| --- | --- |
| Func | Green light display control |
| Param | 1 on 0 off |
| Return | VOID |

  - MQTT

tcp_mqtt_client

|  | int tcp_mqtt_client(Client* client); |
| --- | --- |
| Func | MQTT client settings (TCP) |
| Param | client client |
| Return | 0 succeed other error |

ssl_mqtt_client

|  | I int ssl_mqtt_client(Client* client, const char *ca_cert, uint32_t ca_length); |
| --- | --- |
| Func | MQTT client settings (tls) |
| Param | client client ca_cert cert ca_length cert length |
| Return | 0 succeed other error |

MQTTConnect

|  | int MQTTConnect(Client* c, MQTTPacket_connectData* options)； |
| --- | --- |
| Func | MQTT connection |
| Param | client Client |
| Return | 0 succeed other error |

MQTTSubscribe

|  | int MQTTSubscribe(Client* c, const char* topicFilter, enum QoS qos, messageHandler messageHandler); |
| --- | --- |
| Func | MQTT subscription |
| Param | c client topic Subscription topics qos Quality of publication messageHandler The callback function interface for receiving messages |
| Return | 0 succeed other error |

MQTTPublish

|  | int MQTTPublish(Client* c, const char* topicName, MQTTMessage* message); |
| --- | --- |
| Func | MQTT publish |
| Param | c client topic Subscription topics message The content of the published message |
| Return | 0 succeed other error |

  - File system API

Only list some commonly used ones, please refer to ql_fs. h for others

3.37 ql_fopen

|  | QFILE * ql_fopen(const char *fname, const char *mode); |
| --- | --- |
| Func | Open files in the file system |
| Param | fname file path mode open mode |
| Return | File stream |

3.38 ql_fwrite

|  | int ql_fwrite(void * buffer, size_t size, size_t num, QFILE * fp); |
| --- | --- |
| Func | File write operation |
| Param | buff Written data size buff length num File write format size fp file stream |
| Return | The actual length written |

3.39 ql_fclose

|  | int ql_fclose(QFILE * fp); |
| --- | --- |
| Func | Close the file |
| Param | fp file stream |
| Return | LFS_ERR_OK: no error. other: error |

3.40 ql_fsize

|  | int ql_fsize(QFILE *fp); |
| --- | --- |
| Func | Get file length |
| Param | fp file stream |
| Return | File length |

3.41 ql_fread

|  | int ql_fread(void * buffer, size_t size, size_t num, QFILE * fp); |
| --- | --- |
| Func | Read file |
| Param | buff Read out the content size Read out the content num File write format size fp file stream |
| Return | The actual length read out |

3.42 ql_remove

|  | int ql_remove(const char *fname); |
| --- | --- |
| Func | Remove file |
| Param | fname file path |
| Return | LFS_ERR_OK: no error. other: error |

3.43 ql_copy

|  | int ql_copy(const char *oldpath, const char *newpath); |
| --- | --- |
| Func | Copy file |
| Param | oldpath old file path newpath new file path |
| Return | LFS_ERR_OK: no error. other: error |

3.44 ql_access

|  | int ql_access(const char *path, u32 mode); |
| --- | --- |
| Func | Check if the file exists |
| Param | fname file path mode access pattern |
| Return | LFS_ERR_OK: no error. other: error |

  - Network Multimedia API

3.45 tts_play_set_user_def

|  | int tts_play_set_user_def(char const * usermsg,int BreakEna,int playnow ); |
| --- | --- |
| Func | Network streaming media playback (only supports MP3), this feature only supports playing MP3 files with sampling rates below 44Khz in wifi mode, and supports full sampling rates in 4G mode |
| Param | usermsg: The URL that needs to be played BreakEna：Interrupt flag (reserved interface, not implemented) Playnow：Broadcast priority (1: high 0: low) |
| Return | 0：success 1：fail |

3.46 voice_stop

|  | int voice_stop(void) |
| --- | --- |
| Func | Stop streaming media playback |
| Param |  |
| Return | 0：success -1：fail |

3.47 voice_state

|  | int voice_state(void)； |
| --- | --- |
| Func | Streaming media playback status |
| Param |  |
| Return | 0：No playback 1：playing |

- Button customization instructions

The button customization function is defined by macros   GPIO_BUTTON_USER_DEF (button.c)。

Replace, add, delete key events, and modify them in the ButtonConfig array of key definitions.

The actual key processing is done in the button_queue_ event function, where events can be modified and called functions can be added.