Create OTA package

Firstly, generating an OTA package requires three important files

resource.json This file is a configuration file and does not require manual modification.

Package script:package_resource.ps1/package_firmware.ps1 This is a script file that runs on PowerShell

Original File:

Fireware Original File:: Firmware_Version.bin

Firmware_V0.0.1.bin

Cert and Param Original File:: CertParam_Version.zip

CertParam_V0.0.1.zip

Voice Original File:: Voice_Version.zip

Voice_V0.0.1.zip

The version number is required when naming the file above. The file suffix type should be consistent

- Create OTA firmware package

- Firstly, place the bin file generated in step 4 of project-build.bat into the tms/firmware directory And modify the file name as required.

- Open PowerShell and run package_firmware. ps1. The script requires three parameters to be entered

- Original files that need to be packaged   2 Device Mode  3  Customer Name

- After completing the parameter input, press enter to execute the script, which will automatically generate an OTA firmware package that can be uploaded to the TMS system.

- AP_DS10-S-CN-MQ_DSPREAD_V1.0.6.zip It is a generated OTA firmware package that can be uploaded to the TMS system for updating device firmware.

- Create Cert and Param OTA package

- About tms/CertParam/ ResourceFile/posparam.ini

posparam.ini is JSON fomart.

The following is a list of parameters that support remote updates

| Param Name | Type |  |
| --- | --- | --- |
| MqttUrl | String | MQTT server address |
| MqttPort | String | MQTT server port |
| username | String | Username for connecting to MQTT server |
| password | String | Password for connecting to MQTT server |
| clietId | String | ClientId for connecting to MQTT server In general, no setting is required, SN is used as the client ID by default in the firmware |
| Topic | String | It is the base topic, and the device will automatically append its own SN to form the final subscription topic Example;user/message/sn |
| LogStatus | Int | 0: Release 2:Debug |
| CertState | Int | 0: No certificate needs to be set up 1: A certificate needs to be set up |

- About original Cert and Param files

If set CertState with 0,

posparam.in file is needed to create CertParam_V1.0.6.zip

Please note that the zip file only contains the posparam.ini file Cannot contain other folders

If set CertSate with 1 posparam.iniand cert files are needed to create CertParam_V1.0.6.zip

- Open powershell and run ./CertParam/package_resource.ps1

- Create Cert and Param OTA package.

- Create Voice OTA package

The device supports updating played voice files through TMS. Support single file and batch file updates.

- Orignal voice files: tms/VoiceResource/voice

- Create Voice Original files,

map3 and wav files are needed to create  VoiceResource_V1.0.6.zip

- Open powershell and run package_resource.ps1

- Create Voice OTA package